# RDMA 8-Rail Bandwidth Test Guide

## Aliyun1 and Aliyun3

- Aliyun1: 66.42.116.72

- Aliyun3: 45.76.25.46

## Step 1: Device-to-Subnet Mapping

Run this on both machines to discover which ionic device is wired to which subnet:

for d in /sys/class/infiniband/ionic_*; do

DEV=$(basename $d)

IFACE=$(ls $d/device/net)

IP=$(ip -6 addr show dev "$IFACE" | grep "fd93" | awk '{print $2}' | cut -d/ -f1)

SNET=$(echo $IP | cut -d: -f4)

echo "$DEV $SNET"

done

### Verified Mapping

| Subnet | Aliyun1 Device | Aliyun3 Device |
| --- | --- | --- |
| 150 | ionic_7 | ionic_1 |
| 14f | ionic_0 | ionic_3 |
| 14e | ionic_2 | ionic_0 |
| 14d | ionic_3 | ionic_2 |
| 14c | ionic_4 | ionic_5 |
| 14a | ionic_5 | ionic_4 |
| 14b | ionic_6 | ionic_7 |
| 149 | ionic_1 | ionic_6 |

Devices on both sides must share the same subnet

## Step 2: Clean Slate (Run on BOTH Machines)

sudo pkill -9 ib_write_bw

sleep 2



## Test A: 8-Rail Unidirectional

### Aliyun1 (server) — run first, wait for all 8 "Waiting for client to connect":

sudo pkill -9 ib_write_bw

sleep 2

sudo ib_write_bw -d ionic_7 -x 1 -F -a --report_gbits -p 10000 &

sudo ib_write_bw -d ionic_0 -x 1 -F -a --report_gbits -p 10001 &

sudo ib_write_bw -d ionic_2 -x 1 -F -a --report_gbits -p 10002 &

sudo ib_write_bw -d ionic_3 -x 1 -F -a --report_gbits -p 10003 &

sudo ib_write_bw -d ionic_4 -x 1 -F -a --report_gbits -p 10004 &

sudo ib_write_bw -d ionic_5 -x 1 -F -a --report_gbits -p 10005 &

sudo ib_write_bw -d ionic_6 -x 1 -F -a --report_gbits -p 10006 &

sudo ib_write_bw -d ionic_1 -x 1 -F -a --report_gbits -p 10007 &

### 


Aliyun3 (client) — run after all servers say "Waiting":

sudo pkill -9 ib_write_bw

sleep 2

sudo ib_write_bw -d ionic_1 -x 1 -F -a --report_gbits -p 10000 66.42.116.72 &

sleep 1.5

sudo ib_write_bw -d ionic_3 -x 1 -F -a --report_gbits -p 10001 66.42.116.72 &

sleep 1.5

sudo ib_write_bw -d ionic_0 -x 1 -F -a --report_gbits -p 10002 66.42.116.72 &

sleep 1.5

sudo ib_write_bw -d ionic_2 -x 1 -F -a --report_gbits -p 10003 66.42.116.72 &

sleep 1.5

sudo ib_write_bw -d ionic_5 -x 1 -F -a --report_gbits -p 10004 66.42.116.72 &

sleep 1.5

sudo ib_write_bw -d ionic_4 -x 1 -F -a --report_gbits -p 10005 66.42.116.72 &

sleep 1.5

sudo ib_write_bw -d ionic_7 -x 1 -F -a --report_gbits -p 10006 66.42.116.72 &

sleep 1.5

sudo ib_write_bw -d ionic_6 -x 1 -F -a --report_gbits -p 10007 66.42.116.72 &

## 





Test C: 8-Rail Bidirectional (4 QPs per Rail)

### Aliyun1 (server) — run first, wait for all 8 "Waiting for client to connect":

sudo pkill -9 ib_write_bw

sleep 2

sudo ib_write_bw -d ionic_7 -x 1 -b -q 4 -F -a -n 5000 --report_gbits -p 10000 &

sudo ib_write_bw -d ionic_0 -x 1 -b -q 4 -F -a -n 5000 --report_gbits -p 10001 &

sudo ib_write_bw -d ionic_2 -x 1 -b -q 4 -F -a -n 5000 --report_gbits -p 10002 &

sudo ib_write_bw -d ionic_3 -x 1 -b -q 4 -F -a -n 5000 --report_gbits -p 10003 &

sudo ib_write_bw -d ionic_4 -x 1 -b -q 4 -F -a -n 5000 --report_gbits -p 10004 &

sudo ib_write_bw -d ionic_5 -x 1 -b -q 4 -F -a -n 5000 --report_gbits -p 10005 &

sudo ib_write_bw -d ionic_6 -x 1 -b -q 4 -F -a -n 5000 --report_gbits -p 10006 &

sudo ib_write_bw -d ionic_1 -x 1 -b -q 4 -F -a -n 5000 --report_gbits -p 10007 &

### Aliyun3 (client) — run after all servers say "Waiting":

sudo pkill -9 ib_write_bw

sleep 2

sudo ib_write_bw -d ionic_1 -x 1 -b -q 4 -F -a -n 5000 --report_gbits -p 10000 66.42.116.72 &

sleep 1.5

sudo ib_write_bw -d ionic_3 -x 1 -b -q 4 -F -a -n 5000 --report_gbits -p 10001 66.42.116.72 &

sleep 1.5

sudo ib_write_bw -d ionic_0 -x 1 -b -q 4 -F -a -n 5000 --report_gbits -p 10002 66.42.116.72 &

sleep 1.5

sudo ib_write_bw -d ionic_2 -x 1 -b -q 4 -F -a -n 5000 --report_gbits -p 10003 66.42.116.72 &

sleep 1.5

sudo ib_write_bw -d ionic_5 -x 1 -b -q 4 -F -a -n 5000 --report_gbits -p 10004 66.42.116.72 &

sleep 1.5

sudo ib_write_bw -d ionic_4 -x 1 -b -q 4 -F -a -n 5000 --report_gbits -p 10005 66.42.116.72 &

sleep 1.5

sudo ib_write_bw -d ionic_7 -x 1 -b -q 4 -F -a -n 5000 --report_gbits -p 10006 66.42.116.72 &

sleep 1.5

sudo ib_write_bw -d ionic_6 -x 1 -b -q 4 -F -a -n 5000 --report_gbits -p 10007 66.42.116.72 &

